Minimum Inspection Programme for ELA1 hot-air balloons

To be performed every annual interval.

A tolerance of one month may be applied. However, the next interval shall be calculated from the date originally scheduled (without the tolerance).

Note 1: Use the manufacturer’s maintenance manual to accomplish each task/inspection.

Note 2: Proper operation of backup or secondary systems and components should be included for every instance where a check is performed for improper installation/operation.

- Envelope

| System/component/area | Task & Inspection detail |
| --- | --- |
| Identification (type/serial number/registration plate) | Check for presence and verify type/serial number installed. |
| Crown ring and line | In place; not corroded; crown line undamaged and has appropriate length. |
| Vertical/horizontal load tapes | Check joints with the crown ring, top of the envelope and wires. All load tapes undamaged along their entire length. Inspect base horizontal tape and edge of the envelope top. Inspect joint between base horizontal load tape and vertical load tapes. |
| Envelope fabric | Inspect the envelope fabric panels (including parachute and rotation vents if fitted) for damage, porosity overheating or weakness. Unrepaired damage is within tolerance given by the manufacturer. If substantial fabric porosity is suspected, then a flight test should be performed, but only after a grab test has demonstrated that the balloon is safe to fly. Perform grab test in accordance with the manufacturer’s instructions. |
| Flying cables | Inspect for damage (particularly heat damage). Kevlar cable — yellow core is not visible |
| Karabiners | Inspect for damage. Karabiner lock is working properly. |
| Melting link and Tempilabel | Check maximum temperature indication (flag/‘tell-tale’). |
| Control system lines | Inspect for damage wear, security of knots. Check proper length. Check lines attachments for damage, wear, security. |
| Control lines and their attachments | Inspect for damage, wear, security of knots. Check proper length of the lines. |
| Envelope pulleys | Inspect for damage, wear, free running, contamination, security of attachment. |

- Burner

| System/component/area | Task & Inspection detail |
| --- | --- |
| Identification (type/serial number) | Check for presence and verify type/serial number installed. |
| Burner frame | Inspect welds for cracking. |
|  | Inspect tubes for distortion/deformation/cuts/gouges. |
|  | Inspect frame for security of fasteners (heat shields, flexi-corners). |
|  | Inspect frame lugs for wear, cracking. |
|  | Inspect general condition (corrosion, heat shields). |
| Gimballing | Check stiffness, security of fitting manifolds. |
| Leak check | Perform leak check of the burner. |
| Hoses | Inspect all hoses for wear, damage, leak, and lifetime limitation. Inspect condition and correct function of the fuel. |
| Pressure gauges | Check Pressure gauge reads zero when no pressure applied, lens present. |
| Pilot valves/flame | Check Shut off, free movement, correct function, lubricate if necessary. |
| Whisper valves/flame | Check Shut off, free movement, correct function, lubricate if necessary. |
| Main valves/flame | Check Shut off, free movement, correct function, lubricate if necessary. |
| Coils | Check for damage, distortion, security of fasteners. Inspect welds for cracking. Check security of jets, tighten or replace as necessary. |
| Fuel | Check correct type, check dates (if applicable). |

- Basket

| System/component/area | Task & Inspection detail |
| --- | --- |
| Identification (type/serial number) | Check for presence and verify type/serial number installed. |
| Basket body | Check the general condition of the basket body. Inspect weave for damage, cracks/holes. No sharp objects inside the basket. |
| Basket wires | Inspect for damage, check eye rings. |
| Karabiners | Inspect for damage. Karabiner lock is working properly. |
| Basket floor | Inspect for damage and cracks. |
| Runners | Inspect for damage. |
| Rawhide | Inspect for damage, wear and attachments to the floor. |
| Rope handles | Inspect for damage, security of attachment. |
| Cylinder straps | Inspect for damage, deterioration. |
| Padded basket edge trim | Inspect for damage and wear. |
| Burner rods | Inspect for damage, wear and cracking. |
| Padded burner rod covers | Inspect for damage and wear. |
| Basket equipment | Check presence and functionality. |
| pilot restraint | Inspect for security and condition. |
| Fire extinguisher | Check expiration date and protection cover. |
| First-aid kit | Check for completeness and expiration date. |

- Fuel tanks

| System/component/area | Task & Inspection detail |
| --- | --- |
| Identification (type/serial number) | Check for presence. |
| Cylinder | Check periodic inspections for each cylinder is valid (date) (e.g. 10 years’ inspection). |
| Cylinder body | Inspect for damage, corrosion. |
| Liquid valve | Inspect for damage, corrosion, correct operation. |
|  | Inspect 0-ring seals, lubricate/replace as required. |
| Fixed liquid Level gauge | Inspect for damage, corrosion, correct operation. |
| Contents Gauge | Inspect for damage, corrosion, freedom of movement. |
| Vapour valve | Inspect for damage, corrosion, correct operation (including regulator). |
|  | Inspect Quick Release Coupling for correct operation, sealing. |
| Padded cover | Inspect for damage. |
| Pressure relief valve | Does not indicate over pressuring |
| Assembly | Inspect, leak-test all pressure holding joints using leak detector. |
|  | Functional test |

- Additional equipment

| System/component/area | Task & Inspection detail |
| --- | --- |
| Instruments | Functional check |
| Quick release | Functional check and inspect the condition of the latch, bridle and ropes for wear and deterioration. Check that the karabiners are undamaged and operate correctly. |
| Communication/navigation equipment (radio) | Perform operational check. |
| Transponder | Perform operational check. |