Maintenance Programme Light Aircraft

MPLA / H

Piston Engine Helicopters

# For

## EI-XXX

NOTE: A programme may only be approved for one helicopter type and may not be used for multiple helicopter types. Individual Programmes must be submitted for each helicopter type.

| Helicopter Type |  |
| --- | --- |
| IAA Programme Approval Ref |  |

### NOTE :- Where specific tasks have been mandated by the helicopter Type Certificate Holders, Supplemental Type Certificate Holders, equipment manufacturers, the Irish Aviation Authority or EASA and are not included in this maintenance programme, it is the responsibility of the helicopter owner to insure the required maintenance is performed at the interval specified.

HF

This Maintenance Programme is Human Factors Sensitive

### Contents

Section 0	General

Section 1	Introduction

#### Section 2	Owner / Operator / Subpart G Organisation Certification Statement

Section 3	Responsibilities and Standards

Section 4	Life Items

Section 5	Manufacturer’s Special Instructions

Section 6	Limited Pilot-Owner Maintenance Tasks

Section 7	Variations to the Check Cycle

Section 8	Scheduled Tasks

Section 9	Repetitive Continuing Airworthiness Requirements for

Airframe, Engine and Equipment.

Section 10	Sample Work Sheets

Section 11	Programme Annual Review Checklist.

##### Section 0

General

Helicopter Applicability List

This Helicopter Maintenance Programme is applicable only to the following helicopters,

Table 1

| Registration | Helicopter Serial No. | Engine Type |
| --- | --- | --- |

Distribution List

Copy No. 1		Irish Aviation Authority

Copy No 2.		Owner/Operator (State Name)

Copy No 3.		Maintenance Provider (State Company Name or Individual)

Copy No 4.	Required if managed by a Subpart G Organisation

Record of Amendments

| Revision No | Revision Date | Incorporated By | Incorporation Date |
| --- | --- | --- | --- |
| Issue 1 Rev 1 | Feb 2010 | Original Issue | Feb 2010 |
| Issue 1 Rev 2 | Feb 2013 |  |  |
| Issue 1 Rev 2 | June 2015 |  |  |
| Issue 1 Rev 2 | Jan 2017 |  |  |

All changes in this Programme must be approved by the competent authority or a CAMO that have been granted “Indirect Approval” privileges. If the CAMO approves the changes, the owner of the helicopter must have an agreement with the CAMO in question. The CAMO may only approve changes to this programme in accordance with the procedure in their approved CAME and shall forward an amended copy to the IAA.

List of Effective Pages

| Section | Page No | Issue No | Rev No | Date |
| --- | --- | --- | --- | --- |
| Programme Cover Page | 1 |  |  |  |
| Programme Contents | 2 |  |  |  |
| 0. Section 0 Cover Page | 3 |  |  |  |
| 0. Helicopter Applicability List | 4 |  |  |  |
| 0. Distribution List | 5 |  |  |  |
| 0. Record of Amendments | 6 |  |  |  |
| 0. List of Effective page, | 7 |  |  |  |
| 0. List of Effective page, | 8 |  |  |  |
| 1. Section 1 Cover Page | 9 |  |  |  |
| 1. Introduction | 10 |  |  |  |
| 1. Introduction | 11 |  |  |  |
| 2. Section 2 Cover Page | 12 |  |  |  |
| 2. Owner/Operator Statement | 13 |  |  |  |
| 3. Section 3 Cover Page | 14 |  |  |  |
| 3. Responsibilities / & Standards | 15 |  |  |  |
| 3. Responsibilities / & Standards | 16 |  |  |  |
| 3. Responsibilities / & Standards | 17 |  |  |  |
| 3. Responsibilities / & Standards | 18 |  |  |  |
| 4. Section 4 Cover Page | 19 |  |  |  |
| 4. Life Items | 20 |  |  |  |
| 5. Section 5 Cover Page | 21 |  |  |  |
| 5. Manufacturers Special Inspections | 22 |  |  |  |
| 6. Section 6 Cover Page | 23 |  |  |  |
| 6. Introduction. | 24 |  |  |  |
| 6. Introduction. | 25 |  |  |  |
| 6. Pilot-owner Maintenance Tasks | 26 |  |  |  |
| 7. Section 7 Cover Page | 27 |  |  |  |
| 7. Permitted Variations to the Maint, Programme | 28 |  |  |  |
| 7. Notes on Variations | 29 |  |  |  |
| 8. Section 8 Cover | 30 |  |  |  |
| 8. Introduction | 31 |  |  |  |
| 8. Maintenance Check Cycle | 32 |  |  |  |
| 8. Preflight | 33 |  |  |  |
| 8. Preflight | 34 |  |  |  |
| 8. Preflight | 35 |  |  |  |
| 8. Preflight | 36 |  |  |  |
| 8. Work Pack Cover Page | 37 |  |  |  |
| 8. Final Checks | 38 |  |  |  |
| 8. 50Hr / 6 Months Cover Page | 39 |  |  |  |
| 8. 50Hr / 6 Months Check | 40 |  |  |  |
| 8. 50Hr / 6 Months Check | 41 |  |  |  |
| 8. 50Hr / 6 Months Check | 42 |  |  |  |
| 8. 50Hr / 6 Months Check | 43 |  |  |  |
| 8. 50Hr / 6 Months Check | 44 |  |  |  |

List of Effective Pages (cont)

| Section | Page No | Issue No | Rev No | Date |
| --- | --- | --- | --- | --- |
| 8. 100 Hr Cover Page | 45 |  |  |  |
| 8. 100 Hr Check | 46 |  |  |  |
| 8. 100 Hr Check | 47 |  |  |  |
| 8. 100 Hr Check | 48 |  |  |  |
| 8. 100 Hr Check | 49 |  |  |  |
| 8. 100 Hr Check | 50 |  |  |  |
| 8. Annual Check Cover Page | 51 |  |  |  |
| 8. Annual /Non Align Check | 52 |  |  |  |
| 8. Annual /Non Align Check | 53 |  |  |  |
| 8. Annual /Non Align Check | 54 |  |  |  |
| 8. Annual /Non Align Check | 55 |  |  |  |
| 8. Annual /Non Align Check | 56 |  |  |  |
| 8. Annual /Non Align Check | 57 |  |  |  |
| 8. Annual /Non Align Check | 58 |  |  |  |
| 9. Section 9 Cover | 59 |  |  |  |
| 9. CAR’s Introduction | 60 |  |  |  |
| 9. Worksheet | 61 |  |  |  |
| 9. Worksheet | 62 |  |  |  |
| 10. Section 10 Cover Page | 63 |  |  |  |
| 10. Additional Worksheet | 64 |  |  |  |
| 10. Additional Worksheet | 65 |  |  |  |
| 11. Section 11 Cover Page | 66 |  |  |  |
| 11. Annual Review Checklist | 67 |  |  |  |
| 11. Annual Review Checklist | 68 |  |  |  |

##### Section 1

##### Introduction

Introduction

This programme shall be modified as appropriate by the owner operator / CAMO and may be used for piston engine helicopters below 2,730 kg MTOW. This programme is applicable to EASA regulated helicopters as per (EC) No. 216/2008 Article 4 operating under any of the following categories;

- Private

- Commercial Air Transport,

- Commercial Operations and

- Non – Commercial Air Transport.

This programme is available for download from the IAA website – www.iaa.ie

This Maintenance Programme - Helicopters (MPLA / H) meets the requirement of EASA Regulation (EU) No. 1321/2014 Annex 1 (Part M). However, it is the responsibility of owner operator / CAMO who choose to use this programme as a basis for developing their own individual company or aircraft programme that they review all relevant EASA, IAA, Type Certificate and Supplementary Type Certificate holder requirements to ensure the latest revisions are incorporated into this programme, where appropriate.

In the case of helicopters for which the manufacturer has specified a maintenance programme / schedule, the manufacturer’s programme / schedule may be inserted into Section 8 “Scheduled Tasks” and the generic tasks (1 to 134) removed. Alternatively, the Owner or CAMO may choose to amalgamate the manufacturer’s maintenance programme / schedule into the generic list combining both to define their helicopters maintenance programme.

This Maintenance Programme conforms to the requirements of the Generic Maintenance Programme developed to cover a group of similar types of helicopter. This programme is based on the same type of instructions as the baseline maintenance programme as described in AMC M.A.709.

The helicopter should only be maintained to one approved maintenance programme at a given point in time. Where an owner or operator wishes to change from one approved programme to other, a transfer check or inspection may need to be performed to implement the change.

This programme shall be reviewed annually and amended accordingly when necessary. These reviews shall ensure that the programme continues to be valid in light of the operating experience and instructions from the IAA whilst taking into account new and/or modified maintenance instructions promulgated by the type certificate and supplementary type certificate holders and any other organisation that publishes such data in accordance with Annex (Part-21) to Regulation (EC) No 748/2012.

The programme, and any subsequent amendments, shall be approved by the Irish Aviation Authority (IAA). (M.A.302 (b))

This programme has been formatted in such a way as to provide provision for the owner operator / CAMO to demonstrate compliance with M.A. 302 (d) by compiling the programme through the following;

- By incorporating instructions issued by the Irish Aviation Authority.

- By technically assessing instructions for continuing airworthiness issued by the type-certificate, restricted type-certificate and supplementary type-certificate holders of the Airframe, Engine and Equipment and incorporating, partially incorporating or not incorporating them, as deemed appropriate by the owner / CAMO.

- By incorporating data  published by other organisation when such data is issued under Regulation (EC) No 748/2012  and its annex Part-21 where applicable.

This programme contains details, including frequency of all maintenance to be carried out, including any specific tasks linked to the type and the specific operations. (M.A.302 (e))

The CAMO shall not use the indirect approval procedure if authorised by the IAA when this organisation is not under the oversight of the Member State of Registry, unless an agreement exists in accordance with Part-M Point M.1, Paragraph 4(ii) or 4(iii), as applicable, transferring the responsibility for the approval of the sailplane maintenance programme to the competent authority responsible for the continuing airworthiness management organisation.

HF

Human Factors:

In the preparation of this document, consideration has been given to the Human Factor elements of ICAO Annex I along with EASA Part 66 requirements for helicopter maintenance engineers. Throughout this document, we have included prompts to highlight the importance of considering Human Factors. As is the case with all maintenance tasks, the responsibility lies with the maintenance engineer performing the task or the pilot owner who has elected to perform and certify Limited Pilot Owner Tasks.

HF

Human Factors Prompt =

The absence of such prompts is in no way an indication that Human Factors should not be considered. Human Factors is the responsibility of all who perform and certify maintenance to do every thing within their power to prevent accident and incident to helicopter.

##### Section 2

Owner / Operator / CAMO Certification Statement

###### Owner / Operator / CAMO Certification Statement

###### (Delete as appropriate)

The undersigned undertakes to ensure that the helicopter will continue to be maintained in accordance with this approved maintenance programme. It is understood that non-compliance with any of the responsibilities and standards may affect flight safety and the safe operation of the helicopter and will invalidate the Certificate of Airworthiness.

When preparing this Programme to meet the requirements of Part M, instructions and recommendations made by the airframe, engine and equipment type certificate holders and any supplementary type certificate holder’s have been evaluated and where appropriate have been incorporated.

Where there is conflict between the airframe, engine and equipment type certificate or supplementary type certificate holder’s instructions and recommendations and this generic maintenance programme then the former shall take precedence.

This Programme requires an owner/operator / CAMO to maintain continuing airworthiness records e.g. airframe, engine and where necessary a propeller log book, which will be customised by completing the required continued airworthiness and maintenance details.

In accordance with Part M.A.302 (a), the data contained in the Programme will be reviewed annually for continued validity.

It is accepted that this Programme does not prevent the necessity for complying with any new or amended regulation published by EASA or the Irish Aviation Authority, where these new or amended regulations may override elements of this Programme. If the IAA is no longer satisfied that a safe operation can be maintained the approval of the programme of part of it may be suspended or revoked.

| Name: |  |  | Status : | owner/operator, CAMO (Delete as appropriate) |
| --- | --- | --- | --- | --- |
| Address: |  |  | Contact Telephone No. |  |
| Position: |  |  | Date: |  |
| Signed: For and on behalf of the owner/operator / CAMO: (Delete as appropriate) |  |  |  |  |

Note: Reference should be made to Part M, M.A.201 (a) and (b) for the owner/operator responsibilities.

##### Section 3

Responsibilities and Standards

1.	Owner/Operator Responsibilities

The owner/operator is responsible for the helicopters continuing airworthiness in accordance with Regulation (EU) No. 1321/2014, Appendix 1 (Part M) M.A. 201.

2.	Certificate of Release to Service

On completion of any of the Programme maintenance checks, a detailed, referenced entry must be made in the relevant log book(s), or equivalent, with an appropriate certificate of release to service (CRS) by the certifying person.

CRS for helicopter operated for the purpose of commercial air transport shall be issued by a Part-145 organisation. If the CRS is not issued by a Part 145 organisation it shall be issued by an appropriately rated Subpart F organisation or approved Part 66 licence holder (ref. Part M M.A.801).

For privately operated helicopter of simple design, the pilot-owner may issue CRS in accordance with Part M M.A.803 for maintenance as listed in Part M, Appendix VIII (ref Section 6 of this programme for specific approved tasks).

A CRS is not required subsequent to the completion of the Preflight (or Check A for AMPs approved prior to Revision 3).

3.	Certifying Persons’ Responsibilities

Certifying persons must use their engineering skill and judgment in determining the depth of inspection needed and other matters, which could affect the airworthiness of the helicopter. Certifying persons are responsible for recording in the appropriate logbook or worksheet, any defects, deficiencies or additional maintenance required, resulting from the implementation of the Programme and the issue of the certificate of release to service.

4.	Performance of Maintenance

All maintenance shall be performed in accordance with the methods, techniques, standards and instructions specified in Part M M.A.402.

5.	Airworthiness Life Limitations (Retirement/Scrap Lives)

Airworthiness life limitations shall be those published by the state of design type certificate holder and supplementary type certificate holders. Airworthiness life limitations shall be recorded in the manner specified in section 4 of this Programme or an alternate method acceptable to the IAA.

6.	Airworthiness Directives

Airworthiness directives shall be those issued by EASA, the IAA and the state of design responsible for the type certificate and supplementary type certificates. Compliance with airworthiness directives shall be recorded in the appropriate section of the associated IAA Log Books or any alternative documents or systems acceptable to the IAA.

7.	IAA Generic Requirements

Compliance with IAA Requirements published in Aeronautical Notices shall be recorded in the appropriate section of the associated IAA Log Books or any alternative documents or systems acceptable to the IAA.

8.	Overhaul, Additional Inspections and Test Periods

Overhaul, additional inspections and test periods shall be those recommended by the type certificate holder or supplementary type certificate holders. EASA and the IAA may vary or mandate overhaul and test periods and additional inspections by the issue of an airworthiness directive or IAA Requirements.

9.	Instructions for Continued Airworthiness

Instructions for continued airworthiness consist of in-service data published by the type certificate or supplementary type certificate holder in maintenance manuals, service bulletins, service letters etc. to ensure operational safety and reliability, instructions for continued airworthiness must be formally technically assessed and adopted, as required, by the owner/operator or CAMO.

Assessment of continued airworthiness instructions shall be recorded in the appropriate section of the associated IAA Log Books or any alternative documents or systems acceptable to the IAA. Deviations from ICA, including non-compliance, shall be recorded in the aircraft’s continuing airworthiness system.

10.	Modifications or Repairs

EASA approved modifications or repairs, which have been carried out, must be recorded in the appropriate IAA log book(s) or any alternative documents or systems acceptable to the IAA.

Any additional instructions for continued airworthiness due to modifications or repairs shall be technically assessed and may be recorded in Section 0 of the associated Programme along with inclusion of the specific task in Section 9 “Repetitive Continuing Airworthiness Requirements” (Repetitive CAR’s).

Use of FAA AC43.13-1B. Acceptable Methods, Techniques, and Practices - Aircraft Inspection and Repair and or FAA AC 43.13-2B. Acceptable Methods, Techniques, and Practices - Aircraft Alterations

Currently these documents can only be used during the maintenance of the aircraft listed in this programme when agreed with the Type Certificate Holders. CS-STAN may be used.

11.	Independent Inspections

The type certificate holder or supplementary type certificate holder’s instructions for continued airworthiness should be followed when determining the need for an independent inspection. In the absence of these inspection standards, an independent inspection must be carried out after any flight safety sensitive maintenance task, in accordance with Part M M.A.402 (a) and AMC M.A.402 (a) 4.

12.	Scheduled Maintenance Worksheets

Worksheets shall be issued and each task signed off. These worksheets become part of the maintenance records that must be kept in accordance with Part M M.A.305(h) by the owner/operator. All additional maintenance carried out should be certified on suitably referenced worksheets and included in the helicopter’s records. Scheduled maintenance worksheets and additional worksheets shall be cross-referenced and recorded in the certification areas of the IAA log book(s) or any alternative documents or systems acceptable to the IAA, giving details of airworthiness directives, component changes, scheduled and any additional maintenance carried out.

13.	Defects

Any defect that hazards seriously the flight safety shall be rectified before further flight. Only authorised certifying staff on behalf of a Subpart F or a Part 145 maintenance organisation and / or a Part 66 licence holder can decide, using maintenance data, whether an helicopter defect hazards seriously the flight safety and therefore decide when and which rectification action shall be taken before further flight and which defect rectification can be deferred. Any helicopter defect that would not hazard seriously the flight safety shall be rectified as soon as practicable, after the date the helicopter defect was first identified and within any limits specified in the maintenance data. Any defect not rectified before flight shall be recorded in the helicopter maintenance record system.

14.	Definitions

Throughout the Programme the following terms and abbreviations have the stated definitions;

Service/lubrication (SERVICE/LUB)

The term 'service or lubrication' requires that a component or system should be serviced and/or replenished as necessary with the correct fuel, oil, grease, water, oxygen, etc., to a condition specified in the appropriate maintenance manual. The term may also be used to require filter cleaning or replacement.

Inspect (INSP)

An 'inspection' is a visual check performed externally or internally in suitable lighting conditions from a distance considered necessary to detect unsatisfactory conditions/discrepancies using, where necessary, inspection aids such as mirrors, torches, a magnifying glass etc. Surface cleaning and removal of detachable cowlings, panels, covers and fabric may be required to be able to satisfy the inspection requirements.

Operational check (OP/C)

An 'operational check' is a test used to determine that a system or component or any function thereof is operating normally.

Functional check (F/C)

A 'functional check' is a detailed examination of a complete system, sub-system or component to determine if operating parameters are within limits of range of movement, rate of flow, temperature, pressure, revolutions per minute, degrees of travel, etc., as specified in the appropriate maintenance manual. Measured parameters must be recorded in the associated work pack.

Check (CHK)

A 'check' is the verification of compliance with the type design organisation’s instructions for continuing airworthiness.

Detailed Visual Inspection (DVI)

An intensive visual examination of a specific structural area, system, installation or assembly to detect damage, failure or irregularity. Available lighting is normally supplemented with a direct source of good lighting at an intensity deemed appropriate by the inspector. Inspection aids such as mirrors, magnifying lenses, etc may be use. Surface cleaning and elaborate access procedures may be required.

##### Section 4

Life Limited Items

All items with an overhaul or scrap life as specified by the TC Holder, STC Holder or Equipment manufacturer shall be listed in Table 2 below.

Note: No variation or escalation is allowed on components for which an ultimate (scrap) or Retirement life or an Overhaul limit has been prescribed.

Table 2:

| Item / Component | Type/Make/Model | Part No | Overhaul Life | Scrap Life |
| --- | --- | --- | --- | --- |

Section 5

Manufacturer’s Special Instructions

Special instructions issued by the manufacturer are those additional tasks required by the manufacturer outside of the normal scheduled maintenance tasks listed in Section 8. Depending on the manufacturer these tasks may be classified as special, additional, supplementary or out of phase inspections / maintenance tasks. These inspections / maintenance tasks shall be listed in Table 3 below. The associated task intervals shall also be listed.

Example: Tasks as listed in the manufacturer’s special inspection section of the helicopter, engine or equipment maintenance manual.

Table 3:

| Item No | Manufacturer’s Reference Document | Task to be performed | Frequency |
| --- | --- | --- | --- |

##### Section 6

Limited Pilot-Owner Maintenance Tasks

### NOTE :- A Pilot – Owner shall only certify for tasks listed in this Section which relate to them by name– Limited Pilot Owner Maintenance Tasks (Section 6).

The following is considered the list of Limited Pilot-owner maintenance tasks as specified in Part-M Appendix VIII (ref 1321/2014).

In addition to the requirements laid down in Annex 1 (Part M, ref 1321/2014), the following basic principals are to be complied with before any maintenance task is carried out under terms of the Pilot-owner maintenance.

(a) Competence and responsibility

- The Pilot-owner is always responsible for any maintenance they perform.

- Before carrying out any Pilot-owner maintenance task, the Pilot-owner must satisfy themselves that they are competent to do the task. It is the responsibility of the Pilot-owner to familiarise themselves with the standard maintenance practices for their helicopter and with the helicopter maintenance program. If the Pilot-owner is not competent for the task to be carried out, the task cannot be released by the Pilot-owner.

- The Pilot-owner or their contracted CAMO is responsible for identifying the Pilot-owner tasks according to the basic principals in this maintenance programme and for ensuring that the document is updated in a timely manner.

(b)	Task.

The Pilot-owner may carry out simple visual inspections or operations to check for general condition and for obvious damage and normal operation of the airframe, engine, systems and components.

Maintenance tasks shall not be carried out by the Pilot-owner when the task:

- Is critically safety related, whose incorrect performance will adversely affect the airworthiness of the helicopter or is a flight safety sensitive maintenance task as specified in point M.A.402(a) and/ or,

- Requires the removal of major components or major assembly and/or,

- Is carried out in compliance with an Airworthiness Directive (AD) or an Airworthiness Limitation Item (ALI),  unless specifically allowed in the AD or the ALI and/or,

- Requires the use of special tools, calibrated tools (except torque wrench and crimping tool) and/or,

- Requires the use of test equipment or special testing (e.g. none destructive testing (NDT), system tests or operational checks for avionic equipment) and/or,

- Is composed of any unscheduled special inspections (e.g. heavy landing check) and/or,

- Is effecting systems essential for the IFR operation and/or,

- Is listed in Part M Appendix VII “Complex Maintenance Tasks” (ref 1321/2014) or is a component maintenance task in accordance with point M.A. 502(a),(b),(c) or (d)

The criteria 1 to 8 listed above can not be overridden by less restrictive instructions issued in accordance with “M.A. 302(d) Maintenance”.

Any task described in the helicopter flight manual as preparing the helicopter for flight (Example, assembling the glider wings or pre-flight), is considered a Pilot task and not a Pilot-owner maintenance task and therefore does not require a Certificate of Release to Service.

(c)	Performance of the maintenance Pilot-owner tasks and records

The maintenance data as specified in point M.A.401 must always be available during the conduct of Pilot/Owner maintenance and must be complied with. Details of the data referred to in the conduct of Pilot/Owner Maintenance must be included in the Certificate of Release to Service in accordance with point M.A.803 (d)

The Pilot-owner must inform the approved CAMO responsible for the continuing airworthiness of the helicopter (if applicable) not later than 30 days after completion of the Pilot-owner maintenance task in accordance with point M.A.305 (a)

(d)	Certificate of Release to Service for Pilot-owner maintenance tasks.

The wording of the Certificate of Release to Service for Pilot/Owner maintenance tasks can be found in EASA Regulation 1321/2014 Part M AMC M.A.801(f) 1.(b).

(e)	Ref M.A.803, to qualify as a Pilot-owner, the person must:

1).	hold a valid pilot licence (or equivalent) issued or validated by a Member State for the helicopter type or class rating; and

2)	Own the helicopter, either as sole or joint owner; that owner must be:

(a) One of the natural persons on the registration form; or

(b) a member of a non-profit recreational legal entity, where the legal entity is specified on the registration document as owner or operator, and that member is directly involved in the decision making process of the legal entity and designated by that legal entity to carry out Pilot-owner maintenance.

(f)	Approved Pilot-owner maintenance tasks

The following list submitted by the Pilot-owner or their contracted CAMO is approved under the maintenance programme approval for this helicopter.

Note: Select the appropriate Pilot-Owner task list from the tables included in AMC Appendix VIII and enter the tasks which you elected to perform in the table below.

The tasks listed in Table 4 below specify items that may be completed by a pilot owner who holds a current and valid pilot licence for the helicopter type involved and who meets the competence and responsibility requirements of Appendix VIII to Part-M.

HF

To perform Maintenance on your helicopter you should have all the current maintenance data and tooling available. If you are not fully satisfied that you can competently perform a particular maintenance task for which you have elected to perform and are named below then do not proceed, seek guidance from you maintenance provider.

Limited Pilot-Owner Maintenance Task List;-

This section applies only to privately operated helicopters of simple design with a MTOW of less than 2,730 kg. All Limited Pilot Owner Maintenance Tasks shall be associated with a named Pilot-Owner.

Note: The tasks listed in Table 4 below which are identified by ** shall exclude IFR operations following Pilot / Owner Maintenance. For these helicopters to operate under IFR operations, they shall be released by an appropriate licensed engineer.

| Pilot/Owner Name | License/Rating | License Number | Valid until: |
| --- | --- | --- | --- |

| Task | Pilot/Owner Name | Helicopter Reg |
| --- | --- | --- |

Table 4:

Note:- In relation to Defects, the Pilot / owner may not “Troubleshoot” the defect and can not decide when and which rectification action shall be taken before flight. This must be completed by an appropriately licensed Part 66 engineer or an appropriately approved Subpart F / Part 145 organisation.

Section 7

Variations to the

Check Cycle

Permitted Variations to Inspection Periods (ref. AMC 302(d)).

Option 1: Where the TC/STC holder has not prescribed any variation that may be applied to inspection intervals, the operator may vary the periods prescribed by this Programme provided that such variations are within the limits of Table 5 below.

Option 2: Where the TC/STC holder has prescribed tolerances that may be applied to inspection intervals in the Programme, the operator shall use those tolerance and not those prescribed in Table 5 below.

Note: The Programme must specify which of the above is being used. Variations shall be permitted only when the periods prescribed by this Programme cannot be complied with due to circumstances, which could not reasonably have been foreseen by the operator. Particulars of every variation shall be entered in the appropriate Log Book(s).

Table 5 - Maximum Variation to the Prescribed Inspection Intervals:

| Items Controlled by Flying Hours. | Maximum Variation |
| --- | --- |
| 5,000 flying hours or less | 10% |
| More than 5,000 flying hours | 500 flying hour |
| Items Controlled by Calendar Time | Maximum Variation |
| 1 year or less | 10% or 1 month, whichever is the lesser |
| More than 1 year but not exceeding 3 years | 2 months |
| More than 3 years | 3 months |
| Items Controlled by Landing / Cycles | Maximum Variation |
| 500 landings / cycles or less | 5% or 25 landings / cycles whichever is the lesser |
| More than 500 landings / cycles | 5% or 250 landings / cycles whichever is the lesser |
| Items Controlled by More Than One Limit |  |
| For items controlled by more than one limit, e.g. items controlled by flying hours and calendar time or flying hours and landings/cycles, the more restrictive limit shall be applied. |  |

HF

1 	Permitted variations for tasks controlled by flying hours should not be understood to be a maintenance planning tool, but as an exceptional means to allow the operator to fly for a limited period of time until the required check is performed.

2 	Permitted variations may not be applied to A.Ds, airworthiness life limitations or overhaul and test periods tasks included in the Maintenance Programme, or tasks which  have been classified as mandatory by the Type Certificate / Supplemental Type Certificate holder or the IAA..

3 	Any application of a permitted variation to the maintenance check cycle period must be recorded in the appropriate log book(s) together with the reason for the variation, by a person who is authorised to sign the log book entry for that particular check. Details of the permitted variation must be made visible to the pilot.

4 	Any application of a permitted variation is not required to be deducted from the next scheduled check unless otherwise specified by the TC / STC holder.

Note 1: Piston Engine Overhaul Periods

Please refer to the Irish Aviation Authority Aeronautical Notice A43A at its latest issue for further details.

Note 3: Continuous Airworthiness Management Exposition

CAMO shall specify in the CAME their procedures to assess, apply, control and record variations to the Maintenance Check Cycle for their helicopters.

Applying Variations: (Select one of the 2 options below and delete the other)

Option 1 - Variations to the prescribed inspection intervals will be applied as per the rules of Table 5 of this programme

Option 2 - Variations to the prescribed inspection intervals will be applied as per the tolerances prescribed by the TC / STC holder.

Note: If Option 2 is selected then Table 6 should be amended to reflect the tolerances prescribed by the TC / STC holder.

Section 8

Scheduled Tasks

Introduction:

This section of the MPLA has been developed to provide Owners and CAMOs with a set of generic maintenance tasks that may be used for helicopters for which the manufacturer has not specified a unique maintenance programme / schedule. These tasks, in conjunction with Section 4 “Lifed Items” and Section 5 “Manufacturers Special Inspections” and tasks specified by the IAA and also tasks mandated through Airworthiness Directives (ref. Section 9) combine to define a maintenance programme.

Manufacturer’s Maintenance Programmes / Schedules:

In the case of Helicopters for which the manufacturer has specified a maintenance programme / schedule, the manufacturer’s programme / schedule may be inserted into this Section and the generic tasks removed. Alternatively, the Owner or CAMO may choose to amalgamate the manufacturer’s maintenance programme / schedule into the generic list combining both to define their helicopter’s maintenance programme or to deviate from the manufacturer’s recommended programme.

The Maintenance Check Cycle

For Owners / CAMOs controlling helicopter for which a manufacturer’s programme / schedule is not available the maintenance Check Cycle for the generic list of tasks is in this Section is stated in Table 6.

For Owners / CAMOs controlling helicopters for which a manufacturer’s programme / schedule is available or who choose to amalgamate the manufacturer’s maintenance programme / schedule into the generic list combining both to prescribe their helicopter’s maintenance programme they should delete the generic Maintenance Check Cycle in Table 6 and specify their own Maintenance Check Cycle as prescribed by the associated manufacturer.

The Maintenance Check Cycle (applies only to generic list of tasks, 1 – 134)

Table 6

| Task | Content | Frequency | Reference Document |
| --- | --- | --- | --- |
| Pilot pre-flight | Refer to helicopter flight manual and items A1 to A13 | Prior to every flight | MPLA / H |
| 50 hour check | 50 hour check items | Not exceeding 50 flying hours or 6 months, whichever is the sooner | MPLA / H |
| 100 hour check | 50 and 100 hour check items | Not exceeding 100 flying hours | MPLA / H |
| Annual check | 50, 100 hour and annual check items | Not exceeding 12 months | MPLA / H |

Owner / CAMO shall specify their Maintenance Check Cycle in the table provided above. The items specified in this example are only applicable to the generic list of tasks prescribed in the following pages). If the programme is based on the manufacturer’s recommendations then the items listed above should be deleted and the manufacturer’s maintenance check cycle listed.

Pre-flight checks shall be carried out in accordance with the Helicopter Flight Manual, Pilot’s Operations Handbook, Pilot’s Notes or Operations Manual as appropriate.

Pre Flight – Prior to each flight – Items A1 to A13

| Check Item | Location | Inspection Required | Description of Work to be Performed |
| --- | --- | --- | --- |
| A1 | General | Check | Remove frost, snow or ice, if present. Check that the helicopter documents are available and in order. Ensure all loose equipment is correctly stowed and the helicopter is free of extraneous items. If the helicopter has not been regularly used, ensure before resumption of flying that: - Either the engine has been turned weekly or run fortnightly; or - The manufacturer’s recommendations have been complied with;and - Previously reported defects have been addressed. |
| A2 | Transparencies | Inspect | For damage and for cleanliness. |
| A3 | Fuselage | Inspect Inspect Inspect Inspect | Skin / covering, struts and tubular structure for damage, corrosion and security of all items. Drain holes and vents for freedom from obstruction. Remove pitot head cover/static port blanks, and inspect orifices for cleanliness. Radio aerials for damage and security. |
| A4 | Landing Gear | Check Check Inspect Inspect Check | Shock absorbers, struts for leaks and that extension appears normal. Tyres for inflation, damage and creep. Brake installation for external evidence of leaks and for damage and security. Skid and cross tubes for damage, excessive deflection and worn skid shoes. Ground handling wheels retracted or removed. |

Pre Flight  – Prior to each flight (continued)

| Check Item | Location | Inspection Required | Description of Work to be Performed |
| --- | --- | --- | --- |
| A5 | Flight Controls | Inspect Inspect Check | Flying control surfaces for damage and security of all items and for freedom of operation. Hydraulic cylinders for leakage. Hydraulic reservoir fluid level. |
| A6 | Powerplant/ Engine | Check Inspect Inspect Check Inspect Inspect Check | Oil Level, Security of filler cap and dipstick. Engine, as visible, for leaks, signs of overheating, and security of all items. Engine mounts for damage and security. Fan belts for adequate tension and for condition. Controls for security and damage. Air filter/intake for cleanliness. Security of cowlings, access doors, panels. |
| A7 | Fuel system | Check | Check visually that quantities are compatible with indicator readings. Drain fuel sample from each drain point into a transparent container and check for water, foreign matter and correct colour. |
| A8 | Main Transmission & Rotor | Check Check Inspect Check Inspect Check Check | Transmission for damage, leaks and correct oil level. Freewheel operation. Rotating controls for damage, security and freedom of operation. Levels in oil reservoirs and dampers. Main rotor blades for damage and security; drain holes for freedom from obstruction. Blade inspection monitor. Blade tie downs removed. |

Pre Flight  – Prior to each flight (continued)

| Check Item | Location | Inspection Required |  | Description of Work to be Performed |
| --- | --- | --- | --- | --- |
| A9 | Tail Transmission & Rotor |  | Check Inspect Inspect Check Inspect | Gearbox oil level(s) and for signs of leaks. Rotor assembly for damage, security and freedom of operation. Rotor blades for damage and security. Strike indicators. Drain holes for freedom from obstruction. |
| A10 | Cabin Area |  | Check Check Check Inspect Check Test Inspect Check | Flying and engine controls, for full and free movement in the correct sense. Friction devices for correct operation. Instrument readings are consistent with ambient conditions. Perform manual override & disengagement check on auto-stabiliser system. Avionic equipment operation, using self-test facilities where provided. Seats, belts and harnesses for satisfactory condition, locking and release. Emergency equipment properly stowed and inspection dates valid. Operation of electrical circuits. Cabin and baggage doors for damage, security and for correct operation and locking. Markings and placards are legible. |

Pre Flight  – Prior to each flight (continued)

| Check Item | Location | Inspection Required | Description of Work to be Performed |
| --- | --- | --- | --- |
| A11 | Agricultural operations | Inspect Check NOTE | Hoper, Hopper lid, tank, pump, boom assemblies, pipe runs, blowers and spreaders for damage and security. Emergency dump doors, fan brake and pump control for correct operation. At the earliest opportunity, the helicopter must be completely cleaned to remove chemicals, and an inspection of those parts of the structure which are likely to have been contaminated, e.g. skin/covering and exposed control cables, carried out before the helicopter is returned to service. |
| A12 | Marine helicopter | Inspect Check | Floats, spreaders, struts, bracing wires, for damage, security and corrosion. Fixed float inflation bottles for correct pressure |
| A13 | Special Purpose Equipment | Inspect Check | Emergency flotation gear, lifting hooks, rescue hoists, stretcher installations and similar equipment for damage and security. Lifting hook release operation. |

HF

Consideration should be given by the Pilot / Engineer that the Preflight prior to first flight of the day may be the last inspection to be performed on the helicopter prior to flight. If an item does not look correct then possible action needs to be taken to rectify the situation to prevent an accident or incident occurring.

Work Pack Cover Page

| Maintenance Organisation / Pilot-Owner / AME Name |  |
| --- | --- |
| Approval Reference or AME No: |  |
| Workpack Ref: |  |
| Site where maintenance being accomplished: |  |
| Page 1 of | Note: Enter total pages of Workpack issued |

| Helicopter Registration: EI - |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Type | Serial Number | Total Flying Hours | Hours since new / overhaul |
| Airframe |  |  |  |  |
| Engine |  |  |  |  |

| Check Start Date |  | Check Completion Date |  |
| --- | --- | --- | --- |

| Maintenance Manual Reference Note: Maintenance manuals must be those specified in the maintenance contract. | Issue / Revision No. | Date |
| --- | --- | --- |
| Airframe |  |  |
| Engine |  |  |
| Equipment |  |  |

- All Maintenance Data used must be to the latest revision status.

HF

All tools and ground equipment must be removed from the aircraft following maintenance and accounted for.

- Correct grade of oil and grease used where necessary. All tank caps and covers closed as required.

- If distracted in the performance of a task, consider going back three steps to stop any omission.

- Consider the effects of Complacency, Knowledge, Teamwork, Distractions, Fatigue, Lack of Resources, Pressure, Lack of Assertiveness, Lack of Communication, Norms (deviation from procedure), Stress and Lack of Awareness.

Final Checks (include with all checks except for the Pre-Flight Check

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Ground Run:

| 1 | Engine - Powerplant, liquid, air and gas systems for leaks during and following ground run. | INSP | All Checks |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 | Instruments, systems and services. Radio for electromagnetic interference. | OP/C | All Checks |  |  |
| 3 | Engine - Following ground run, ensure all cowlings, access panels and doors are secure. | CHK | All Checks |  |  |

Certification:

| 4 | Workpack and Log Book entries have been completed and certified. Ensure items due in accordance with the relevant approved IAA Log Books or the alternative document or system acceptable to the IAA have been accomplished and certified. | CHK | All Checks | N / A |  |
| --- | --- | --- | --- | --- | --- |

Type Certificate and Schedule Review:

| 5 | Helicopter complies with the type certificate data sheet. | CHK | Annual | N / A |  |
| --- | --- | --- | --- | --- | --- |
| 6 | Mandatory placards are installed and legible. | CHK | Annual | N / A |  |

Notes:

1. Certifying Person Refer to Section 3

2. Inspectors must be proven competent to ensure that all required maintenance tasks are carried out and where not completed or where it is evident that a particular maintenance task cannot be carried out to the maintenance data, then such problems will be reported to the certifying person for appropriate action.

3. Performers must be proven competent to carry out maintenance tasks to any standard specified in the maintenance data and will notify supervisors of defects requiring rectification to re-establish required airworthiness standards.

MPLA / H

50 Hour / 6 Months Check

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Structural / Zonal:

| 7 | External structure of cabin, centre section, tailboom, cowlings, nacelles and stabilisers. | INSP | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 8 | Normal and emergency doors and windows, door hinges, door hinge attachment points, required placards and operating instructions. | INSP | 50 FH / 6 months |  |  |
| 9 | Doors, hatches and windows latching and locking. | OP/C | 50 FH / 6 months |  |  |
| 10 | Agricultural Installations: Hopper, hopper lid, tank, pump, fan, boom assemblies, pipe runs, blowers and spreaders. | INSP | 50 FH / 6 months |  |  |
| 11 | Agricultural Installations: Emergency dump doors, fan brake and pump control. | OP/C | 50 FH / 6 months |  |  |
| 12 | Marine Helicopters: Floats, spreaders, struts and bracing wires. | INSP | 50 FH / 6 months |  |  |

Landing Gear:

| 13 | Landing gear assemblies, shock-absorber struts/units for leaks and correct extension, brake system, brake linings, drums/discs, wheels and tyres. | INSP | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 14 | Tyre pressures, hydraulic brake system fluid level. | Service | 50 FH / 6 months |  |  |
| 15 | Cross tubes for excessive deflection and skid shoes. | INSP | 50 FH / 6 months |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Flying Controls:

| 16 | Primary/secondary flight controls and trim systems for full and free movement in the correct sense and friction devices for correct operation. Position indicators agree with surface movement. | OP/C | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |

Liquid, Air and Gas Systems:

| 17 | Hydraulic, pneumatic, vacuum and other fluid systems. | INSP | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 18 | Fluid levels in reservoirs and accumulator pressures. | Service | 50 FH / 6 months |  |  |
| 19 | Pitot/static system vents, pitot head and drains clear. Pitot head correctly aligned. | INSP | 50 FH / 6 months |  |  |

Equipment and Environmental:

| 20 | Correct stowage of equipment, validity of date on emergency equipment. | CHK | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 21 | Seats, belts/harnesses, attachment, locking and release. | INSP | 50 FH / 6 months |  |  |
| 22 | Fire extinguisher for leakage or discharge. | CHK | 50 FH / 6 months |  |  |

Transmission Installation:

| 23 | Gearboxes, driveshafts, flexible couplings, belts, mast assembly, rotor brake, mountings, support bearings, clutch and freewheel. | INSP | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 24 | Magnetic plugs, system hoses and vents. | INSP | 50 FH / 6 months |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Rotor:

| 25 | Main rotor head and blades assembly. | INSP | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 26 | Tail rotor hub and blades assembly. | INSP | 50 FH / 6 months |  |  |
| 27 | Main and tail rotors freedom of movement in all planes. | CHK | 50 FH / 6 months |  |  |

Helicopter Lubrication:

| 28 | Lubricate helicopter in accordance with type design organisation recommendations. | LUB | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |

Powerplant Installation:

| 29 | Engine controls for full and free movement – throttle, mixture and carburettor heat. | OP/C | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 30 | Powerplant installation. | INSP | 50 FH / 6 months |  |  |
| 31 | Engine cooling fan installation. | INSP | 50 FH / 6 months |  |  |

Air Induction:

| 32 | Air filter, intake and induction system and turbocharger impeller. | INSP | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |

Exhaust:

| 33 | Exhaust manifold, mufflers. | INSP | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Engine Lubrication:

| 34 | Magnetic plugs. | CHK | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 35 | Engine oil change. Oil filter. Screen. Note: In accordance with type design organisation recommendations. Next due: | Service | 50 FH or see Note. |  |  |

Fuel System:

| 36 | Filters for cleanliness and tank vents unobstructed. Drain samples from all drain points and check for presence of water, foreign matter and correct colour. | CHK | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |

Electrical System:

| 37 | Battery, stowage/compartment, vents and drains. Electrolyte level. | INSP & Service | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 38 | Alternator/generator drive belt tension and condition. | INSP | 50 FH / 6 months |  |  |

Radio:

| 39 | Placards and markings legible. | INSP | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 40 | VHF ground function. | OP/C | 50 FH / 6 months |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Instrument Systems:

| 41 | Instruments. Legibility of markings and associated placards, band ranges and limit markings. | INSP + CHK | 50 FH / 6 months |  |  |
| --- | --- | --- | --- | --- | --- |
| 42 | Readings consistent with ambient conditions. | INSP | 50 FH / 6 months |  |  |

MPLA / H

100 Hour Check (includes 50 Hour check items):

Perform 50 Hour check items and the following

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Structural/Zonal:

| 43 | Internal structure of cabin centre section, tail boom, engine bay and transmission platform. Floors, bulkheads, pylons, structural attachment joint assemblies and ballast weight attachments. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 44 | Internal structure of stabiliser and fin assemblies. | INSP | 100 FH |  |  |
| 45 | Internal corrosion protective treatments, drain holes and paths. | INSP | 100 FH |  |  |
| 46 | Static discharge wicks and attachment bases. | INSP | 100 FH |  |  |

Landing Gear:

| 47 | Structural members, attachment fittings, pivot points, shock absorbing devices, main wheels, nose wheel, bearings, skids, hoses and lines. Note: Carry out with weight off the landing gear. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 48 | Parking brake. | OP/C | 100 FH |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Flying Controls:

| 49 | Hinges, brackets, push-pull rods, bellcranks, damper weights, control horns, cables, pulleys, chains, tubes, guides, fairleads and servo-actuators. Note: The need for removal of flying control cables and control system components for detailed inspection must be assessed when accomplishing this task at the annual check. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 50 | Turnbuckles, locking devices in safety. | CHK | 100 FH |  |  |
| 51 | Rotating control assemblies, stabiliser bars, dampers and swashplates. | INSP | 100 FH |  |  |
| 52 | Control locking systems. | OP/C | 100 FH |  |  |

Liquid, Air and Gas systems:

| 53 | Tanks, powerpacks, valves, pipelines, hoses, actuators, filters and venturis. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |

Equipment and Environmental:

| 54 | Cabin air system, heater and blower. | INSP & OP/C | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 55 | Air conditioner, oil level. | OP/C & Service | 100 FH |  |  |

Transmission Installation:

| 56 | Lubrication system, tank, sump, cooler and pipelines. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Powerplant Installation:

| 57 | Crankcase, accessory housings, cylinder assemblies, accessory drive belts, accessories, engine shock mounts, mount frames, bulkheads, firewalls and sealing, cooling baffles, cowlings, breathers and vents and items in engine bay for mutual interference. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 58 | Valve operating mechanism. Note: In accordance with type design organisation recommendations. Next due: | CHK | 100 FH or see Note |  |  |
| 59 | Cylinder compression and leakage. Record results below. Method: | CHK | 100 FH |  |  |

| Eng Cyl | Result | Eng Cyl | Result |
| --- | --- | --- | --- |
| 1 |  | 4 |  |
| 2 |  | 5 |  |
| 3 |  | 6 |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Air Induction:

| 60 | Carburettor heat, alternative air bypass doors and control system. | INSP & OP/C | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 61 | Flame traps, drains. | INSP | 100 FH |  |  |

Ignition:

| 62 | Magnetos, harnesses, leads, switches, starting vibrators, contact breakers, cooling system and ventilators. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 63 | Magneto internal timing and timing to engine. | CHK | 100 FH |  |  |
| 64 | Magneto cam. Note: In accordance with type design organisation recommendations. Next due: | LUB | 100 FH or see Note |  |  |
| 65 | Spark plugs. Note: In accordance with type design organisation recommendations. Next due: | CHK | 100 FH or see Note |  |  |

Exhaust:

| 66 | Cabin heat exchanger. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 67 | Turbocharger, control system, pipelines and hoses. | INSP | 100 FH |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Engine Lubrication:

| 68 | Tanks, sumps, coolers, hoses, pipelines and vents. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 69 | Engine controls in accordance with type design organisation recommendations. | LUB | 100 FH |  |  |

Fuel System:

| 70 | Tanks, filler caps, selector valves, pumps, pipelines, hoses, carburettor, injector systems, throttle, mixture control, fuel selector control and filler point placard. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |

Electrical System:

| 71 | Components, wiring, terminals and connectors. | INSP | 100 FH |  |  |
| --- | --- | --- | --- | --- | --- |
| 72 | Warning circuits. | OP/C | 100 FH |  |  |
| 73 | Correct type and rating of fuses and circuit breakers. Correct spare fuses carried. | CHK | 100 FH |  |  |
| 74 | Lamps and lighting. Correct spare lamps carried. | CHK | 100 FH |  |  |
| 75 | Brushes in starter, alternator and generator. Note: In accordance with type design organisation recommendations. Next due: | CHK | 100 FH or see Note |  |  |

MPLA / H

Annual Check + Non-Aligned Tasks

(includes 50 and 100 hour check items):

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Structure / Zonal

| 76 | Emergency exit by internal and external release methods. | OP/C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 77 | Lightning strike bonding. | CHK | Annual |  |  |
| 78 | Internal condition of struts, control tubes and similar hollow members. Note: In accordance with type design organisation recommendations. Next due: | INSP | See Note |  |  |

Flying Controls:

| 79 | Control cables for correct tension. Control neutrals and travels. Record results below. | CHK | Annual |  |  |
| --- | --- | --- | --- | --- | --- |

| Cable Identification | TEMP | Required Tension | Actual Tension |
| --- | --- | --- | --- |

| Control and position (neutral, min, max etc.) | Required Angle / Measurement | Actual Angle / Measurement |
| --- | --- | --- |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Liquid, Air & Gas Systems:

| 80 | Pitot / static system sense and leak. | F / C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 81 | Hydrostatic test of pressure vessels. Note: In accordance with type design organisation recommendations. Next due: | INSP & CHK | 60 months or see Note |  |  |
| 82 | Flexible fuel and oil hoses pressure test. Note: In accordance with type design organisation pressure testing recommendations but in either case only until the ultimate service life, if stated, is achieved. Next due: | CHK | 72 months from new, then every 36 months or see Note |  |  |
| 83 | Internal examination and pressure testing of fluid tanks and reservoirs. Note: In accordance with type design organisation recommendations. Next due: | CHK | See Note |  |  |

Transmission Lubrication:

| 84 | Transmission oil change. Oil filter and screens. Note: In accordance with type design organisation recommendations. Next due: | Service | 100 FH or see Note |  |  |
| --- | --- | --- | --- | --- | --- |

Equipment & Environmental:

| 85 | Fire extinguisher contents by pressure / weight. | CHK | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 86 | Combustion heater. Note: In accordance with GR 11. Next due: | CHK | I.A.W. UK CAP 747 GR 11 |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Exhaust:

| 87 | Cabin heat exchanger pressure test. Note: In accordance with type design organisation recommendations. Next due: | CHK | Annual or see Note |  |  |
| --- | --- | --- | --- | --- | --- |

Electrical Systems:

| 88 | Over/under-volt system, warnings. Load sharing. | OP / C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 89 | All ground operable electrical circuits. Exercise manually operated circuit breakers. | OP / C | Annual |  |  |
| 90 | Nickel-cadmium battery capacity test. Note: In accordance with equipment manufacturer’s recommendations where capacity checks are recommended by the equipment manufacturer. Next due: | F / C | 12 months or see Note |  |  |
| 91 | Lead-acid battery capacity test. Note: In accordance with equipment manufacturer’s recommendations where capacity checks are recommended by the equipment manufacturer. Next due: | F / C | 12 months or see Note |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Avionics Radio:

| 92 | HF Communication. | OP / C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 93 | ADF ground function using station(s) of known bearing to establish accuracy. Audio on all bands. | F / C | Annual |  |  |
| 94 | ILS Localiser and Glide Slope using a field test set, including flag warnings of single tone failure, centre-line accuracy, sense, course widths and audio. | F / C | Annual |  |  |
| 95 | VOR using a field test set, including flag warnings, omni-radial resolving, radio-magnetic indicator accuracy at 134° intervals, sense and course width. | F / C | Annual |  |  |
| 96 | Marker using a field test set, including 3-tone operational check and high/low sensitivity. | F / C | Annual |  |  |
| 97 | DME using a field test set, including frequency tolerance, range accuracy and audio. | F / C | Annual |  |  |
| 98 | ATC Transponder using a field test set, including frequency tolerance, side lobe suppression, mode 'C' and 'S'. Note: The Mode 'S' checks should confirm that the helicopter assigned Mode 'S' code is correct and that any declared parameters are correct. | F / C | Annual |  |  |
| 99 | Airborne search and weather radar in all modes. | OP / C | Annual |  |  |
| 100 | Area and satellite navigation (GPS). | OP / C | Annual |  |  |
| 101 | Audio control panel, including emergency operation. | OP / C | Annual |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Avionics Radio:

| 102 | ELT, including battery. Note: In accordance with equipment manufacturer’s recommendations. Next due: | CHK | See Note |  |  |
| --- | --- | --- | --- | --- | --- |
| 103 | VHF Communication using a field test set, including frequency tolerance of transmitted frequencies. Note: In accordance with equipment manufacturer’s recommendations, only where frequency tolerance checks are recommended by the equipment manufacturer. Next due: | F / C | Annual |  |  |
| 104 | HF Communication system using a field test set, including frequency tolerance of transmitted frequencies. Note: In accordance with equipment manufacturer’s recommendations, only where frequency tolerance checks are recommended by the equipment manufacturer. Next due: | F / C | 36 months |  |  |
| 105 | Aerials and Feeders – VSWR (DME and ATC Transponder), insulation (HF). Next due: | F / C | 36 months |  |  |
| 106 | Aerials, insulators, controllers, instruments, displays, microphones, headsets, jackplugs and sockets. | INSP | Annual |  |  |
| 107 | Cables and terminals, cooling systems and moisture trap areas. | INSP | Annual |  |  |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Instrument Systems:

| 108 | Air Speed Indicator calibration (in situ is permissible). Note: Measured parameters must be recorded. | F / C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 109 | Altimeter calibration check (in situ is permissible). Note: Measured parameters must be recorded. | F / C | Annual |  |  |
| 110 | Instruments and indicators for satisfactory condition, mounting, marking and operation. Note: This task is applicable to all instruments and indicators that could affect the airworthiness or operating safety of the helicopter. | F / C | Annual |  |  |
| 111 | Compass 'deviation' or 'steer by' cards – valid until next check. | CHK | Annual |  |  |
| 112 | Instruments, displays, controllers, panels, mounts, pipes, hoses, electrical wiring, gyro filters, flux detectors and instrument transmitters. | INSP | Annual |  |  |
| 113 | Compass swing. Next due: | F / C | 36 months |  |  |

Avionics Auto - Pilot / Stabiliser:

| 114 | Auto-Pilot/Stabiliser in all modes including manual override disengagement functions. | OP / C | Annual |  |  |
| --- | --- | --- | --- | --- | --- |
| 115 | Displays, instruments, controllers. | INSP | Annual |  |  |
| 116 | Auto-pilot computer, amplifier, power supply, servo motors, connections to flying control system, automatic trim system, yaw dampers and manometric system inter-connections. | INSP | Annual |  |  |

| Task No | Out of Phase Avionic Checks | Interval | Performed | Date |
| --- | --- | --- | --- | --- |
| 1 | Calibration check of altitude encoder per IAA AN 39 latest issue (AOC aircraft and IFR operations only) | 1 Year |  |  |
| 2 | VOR/LOC/GS check with a filed test set including flag warnings, centerline accuracy, omni–radial resolving, sense and course width, RMI accuracy at 90 degree intervals and audio. | 1 Year |  |  |
| 3 | ADF check on ground using stations of known bearing to establish accuracy and audio check of all bands | 1 Year |  |  |
| 4 | GPS ground function check | 1 Year |  |  |
| 5 | Interphone and audio system check including emergency system and visual inspection of headsets and microphones where fitted | 1 Year |  |  |
| 6 | Carry out Battery Capacity check *(1 year from new and then at 6 month intervals) | 6 Months* |  |  |

Section 9

Repetitive Continuing

Airworthiness Requirements for

Airframe, Engine and Equipment

Repetitive Continuing Airworthiness Requirements for Airframe, engine and equipment.

When an AD, SB, Modification, Repair or STC requiring repetitive maintenance actions has been embodied on the helicopter, engine or equipment listed in this programme (ref section 0 Applicability) the AD, SB, Modification, Repair or STC shall be listed in table 1 below.

All repetitive maintenance tasks associated with ADs, SBs, Modifications, Repairs and STCs and listed in Table 7 below and shall be further described by Task Description, Task Code and Task Interval on the following pages.

Table 7:

|  |  |  | Helicopter Registration |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Originating Document (AD, SB, Mod, Repair, STC) | Date | Revision | EI-ABC |  |  |  |  |

| Document Reference | Task Description | Task Code | Task Interval |
| --- | --- | --- | --- |

| Document Reference | Task Description | Task Code | Task Interval |
| --- | --- | --- | --- |

Section 10

Sample / Additional Worksheets

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

| Task No. | Task Description | Task Code | Task Interval | Performed | Inspected |
| --- | --- | --- | --- | --- | --- |

Section 11

Annual review check list

| Programme Annual review check list (page 1 of 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date: |  | Programme approval Ref. |  |  |  |
| Annual review No: |  | Programme revision status when reviewed. |  |  |  |
| Task | Programme Ref | Prompt |  | Not OK | OK |
| 1 | Cover Page | Check Helicopter type, registration or Sub Part G Organisation. Check IAA approval reference. |  |  |  |
| 2 | Section 0 | Check A/C registration, Programme Revision status. Check Competent Authority or CAMO approval details. |  |  |  |
|  | Section 0 | Check contents pages and compare with programme. |  |  |  |
|  | Section 0 | Check helicopter applicability list including engine and propeller types. |  |  |  |
|  | Section 0 | Check that programme distribution list includes all interested parties including contracted organisations. |  |  |  |
|  | Section 0 | Check that revision status page is updated. |  |  |  |
| 3 | Section 0 | Check List of effective pages. |  |  |  |
| 4 | Section 1 | Check all stated references |  |  |  |
| 5 | Section 2 | Check for correct details and signature of Owner / Operator or Sub Part G Organisation as applicable. |  |  |  |
| 6 | Section 3 | Check all stated references |  |  |  |
| 7 | Section 4 | Check list of Lifed Items for completeness. |  |  |  |
|  |  | Check all part numbers quoted are correct. |  |  |  |
|  |  | Check all makes / models quoted are correct. |  |  |  |
|  |  | Check all overhaul / scrap life quoted are correct. |  |  |  |
| 8 | Section 5 | Check all manufacturers’ special instructions have been included. |  |  |  |
|  |  | Check the frequency for each task is as per the manufacturer’s instructions. |  |  |  |
| 9 | Section 6 | Check all stated references |  |  |  |
|  |  | Check the validity of licences for all pilot owners listed to insure the licence does not expire before the next programme review date. |  |  |  |
|  |  | Check tasks listed versus Part M Appendix VIII of regulation 1321/2014. |  |  |  |
| 10 | Section 7 | Check that only one option for variations has been stated. |  |  |  |
|  |  | Check if the option to use TC / STC holder’s tolerances has been selected then they are stated in Table 6. |  |  |  |
| Programme Annual review check list (page 2 of 2) |  |  |  |  |  |
| Task | Programme Ref | Prompt |  | Not OK | OK |
| 11 | Section 8 | Check, as applicable, that the Maintenance Check Cycle as stated is as per the Generic requirements of the IAA programme or as per the Manufacturer’s stated requirements. |  |  |  |
|  |  | Check, as applicable, that all maintenance tasks are included in the IAA Generic Programme or that all the Manufacturer’s requirements are included in the scheduled tasks list. |  |  |  |
| 12 | Section 9 | Check that all continuous airworthiness requirements (CAR’s) are listed. (AD, SB, Mod, Repair, STC) |  |  |  |
|  |  | Check that the applicable document is referenced at its current revision status. |  |  |  |
|  |  | Check that the Task Code and Task Intervals are as per the instructions listed in the associated document. |  |  |  |
| 13 | Section 10 | Check all stated references. |  |  |  |
| 14 | Section 11 | Check that all sections of this check list have been completed. |  |  |  |
|  |  | File this checklist with the associated Maintenance Programme. |  |  |  |
|  |  | NOTES: |  |  |  |
| CAMO: Owner/Operator: |  |  | Date of Review: |  |  |
| Reviewed By: |  |  | Signature: |  |  |